PTU-Based Laser Pointing System

From Reactive to Predictive Tracking — Complete Project Plan & Pipeline

# SECTION 1: PROJECT OVERVIEW

## 1.1 Objective

Replace the existing reactive “reduce-the-distance” PTU tracking logic with a predictive, Kalman Filter-based algorithm to achieve ≥80% laser-on-drone time during active movement.

## 1.2 Existing Infrastructure (Do Not Modify)

- Working UI (User Interface)

- Camera integration with 36x optical zoom

- PTU (Pan-Tilt Unit) hardware control

- YOLOv8-based drone detection pipeline

- Basic reactive tracking loop

## 1.3 Success Criteria

| Metric | Target |
| --- | --- |
| Drone speed handled | Up to 1 m/s |
| Drone distance | Up to 50 meters |
| Laser on-target rate | ≥80% during movement |
| System latency | Minimized to meet 80% target |

# SECTION 2: CURRENT SYSTEM ANALYSIS (Phase 1)

## 2.1 Reactive Tracking — The Problem

Current logic (pseudo-code):

loop: bbox = yolo_detect(frame) dx = bbox.center_x - crosshair_x dy = bbox.center_y - crosshair_y ptu.move(step * sign(dx), step * sign(dy))

Problems:

- No velocity estimation — the PTU always reacts to where the drone WAS

- Constant step size causes oscillation near target

- No prediction — laser lags behind moving drone

- No Kalman smoothing — noisy detections cause jitter

## 2.2 Latency Sources to Measure

- YOLO inference time (typically 20–50 ms on GPU, 80–150 ms on CPU)

- Frame capture/buffer delay

- PTU command round-trip time (serial/TCP)

- Python GIL contention in main loop

## 2.3 Analysis Tasks

- Instrument the existing loop with timestamps at each stage

- Log: frame_time, detection_time, ptu_command_time, ptu_ack_time

- Calculate total lag = (ptu_ack_time) - (drone_actual_position_time)

- Identify the dominant bottleneck

# SECTION 3: IMPLEMENTATION PLAN (Phase 2)

## 3.1 Architecture Overview

The new pipeline:

Camera Frame → YOLOv8 Detection → Kalman Filter → Predictive Lookahead → Coordinate Transform → PTU Command

## 3.2 Component 1 — Kalman Filter (State Estimator)

State vector: [x, y, vx, vy] (pixel position + velocity)

The Kalman Filter will:

- Estimate the drone’s true position from noisy YOLO bounding boxes

- Predict the drone’s state one step ahead (even when detection fails)

- Smooth out jitter from YOLO fluctuations

Key equations:

- Predict: x̂ = F·x + B·u (state transition)

- Update: x̂ = x̂ + K·(z - H·x̂) (measurement correction)

Recommended library: filterpy (pip install filterpy)

## 3.3 Component 2 — Predictive Lookahead

After the Kalman Filter estimates [x, y, vx, vy], aim at the FUTURE position:

lookahead_time = system_lag_ms / 1000.0 # measured in Phase 1 predicted_x = kalman_x + vx * lookahead_time predicted_y = kalman_y + vy * lookahead_time

The lookahead_time compensates exactly for the measured system lag.

## 3.4 Component 3 — Coordinate Transformation

Convert pixel coordinates to PTU pan/tilt angles:

pan_angle = (predicted_x - frame_center_x) * (H_FOV / frame_width) tilt_angle = (predicted_y - frame_center_y) * (V_FOV / frame_height)

Where:

- H_FOV = horizontal field of view (degrees) at current zoom level

- V_FOV = vertical field of view (degrees) at current zoom level

- These must be calibrated per zoom level (36x zoom ≈ ~0.5° × 0.3° FOV)

## 3.5 Component 4 — Adaptive Step / PID Controller (Optional Enhancement)

Replace fixed step moves with a PID controller:

error_pan = target_pan - current_pan error_tilt = target_tilt - current_tilt ptu_pan_cmd = Kp*error_pan + Ki*integral_pan + Kd*derivative_pan ptu_tilt_cmd = Kp*error_tilt + Ki*integral_tilt + Kd*derivative_tilt

Benefits:

- Proportional: faster response to large errors

- Integral: eliminates steady-state offset

- Derivative: damping to prevent overshoot

# SECTION 4: COMPLETE CODE PIPELINE

## 4.1 File Structure

project/ ├── tracker/ │ ├── __init__.py │ ├── kalman_tracker.py ← Kalman Filter wrapper │ ├── predictor.py ← Lookahead logic │ ├── coord_transform.py ← Pixel → PTU angle │ └── pid_controller.py ← Optional PID ├── main_tracking_loop.py ← Replace existing loop └── config.py ← All tunable parameters

## 4.2 kalman_tracker.py

import numpy as np from filterpy.kalman import KalmanFilter class DroneKalmanTracker: def __init__(self, dt=0.033): # dt = 1/30 fps self.kf = KalmanFilter(dim_x=4, dim_z=2) self.dt = dt # State: [x, y, vx, vy] self.kf.F = np.array([[1,0,dt,0], [0,1,0,dt], [0,0,1, 0], [0,0,0, 1]]) # Measurement: [x, y] self.kf.H = np.array([[1,0,0,0], [0,1,0,0]]) self.kf.R *= 10 # Measurement noise self.kf.Q *= 0.1 # Process noise self.initialized = False def update(self, cx, cy): if not self.initialized: self.kf.x = np.array([cx, cy, 0, 0]) self.initialized = True else: self.kf.predict() self.kf.update(np.array([cx, cy])) return self.kf.x # [x, y, vx, vy] def predict_only(self): """Call when detection fails -- extrapolate position""" self.kf.predict() return self.kf.x

## 4.3 predictor.py

class PredictiveLookahead: def __init__(self, lag_seconds=0.1): self.lag = lag_seconds # Measured system lag def get_target(self, state): x, y, vx, vy = state target_x = x + vx * self.lag target_y = y + vy * self.lag return target_x, target_y

## 4.4 coord_transform.py

class CoordTransform: def __init__(self, frame_w, frame_h, h_fov_deg, v_fov_deg): self.fw = frame_w self.fh = frame_h self.h_fov = h_fov_deg self.v_fov = v_fov_deg def pixel_to_angle(self, px, py): cx, cy = self.fw / 2, self.fh / 2 pan = (px - cx) / self.fw * self.h_fov tilt = (py - cy) / self.fh * self.v_fov return pan, tilt

## 4.5 main_tracking_loop.py (New Logic)

tracker = DroneKalmanTracker(dt=1/fps) predictor = PredictiveLookahead(lag_seconds=measured_lag) transform = CoordTransform(1920, 1080, h_fov=0.5, v_fov=0.3) while True: frame = camera.get_frame() detections = yolo.detect(frame) if detections: cx, cy = get_bbox_center(detections[0]) state = tracker.update(cx, cy) else: state = tracker.predict_only() # Coasting if tracker.initialized: target_x, target_y = predictor.get_target(state) pan, tilt = transform.pixel_to_angle(target_x, target_y) ptu.move_absolute(pan, tilt)

# SECTION 5: OPTIMIZATION (Phase 3)

## 5.1 Python Performance Optimizations

- Run YOLO inference in a separate thread/process to avoid blocking the PTU command loop

- Use a double-buffer: YOLO thread writes detection; main thread reads + sends PTU commands

- Use numpy for all matrix operations (already used by filterpy)

- Set Python process priority to high (os.nice(-10) on Linux)

- Use asyncio or threading.Thread for non-blocking PTU communication

## 5.2 Threading Architecture

Thread 1 (YOLO): camera → yolo_detect → write to shared_detection (lock-free ring buffer) Thread 2 (Control): read shared_detection → kalman_update → predict → ptu_command

## 5.3 Kalman Tuning Parameters

| Parameter | Effect | Tune When |
| --- | --- | --- |
| R (measurement noise) | Higher = trust model more | YOLO detections are noisy/jittery |
| Q (process noise) | Higher = trust measurements more | Drone changes direction suddenly |
| dt (time step) | Must match actual frame rate | FPS changes |
| lag_seconds | Lookahead compensation | Measured system lag changes |

# SECTION 6: TESTING & VALIDATION

## 6.1 Test Protocol

- Record ground truth video with known drone trajectory (ruler-measured movement at 1 m/s)

- Run new tracking code on recorded video

- For each frame: log (laser_position, drone_position, on_target = distance < threshold)

- Calculate success_rate = frames_on_target / total_frames

## 6.2 Success Metrics

| Test | Pass Condition |
| --- | --- |
| Static drone | 100% on-target within 2 seconds |
| Slow linear (0.3 m/s) | ≥90% on-target |
| Fast linear (1 m/s) | ≥80% on-target |
| Direction change | ≤0.5s recovery time |
| Detection dropout (0.5s) | Resumes tracking after re-detection |

## 6.3 Validation Script (pseudo-code)

results = [] for frame, gt_pos in zip(video_frames, ground_truth): detections = yolo.detect(frame) # ... run tracker ... laser_pos = ptu.get_current_position() on_target = distance(laser_pos, gt_pos) < THRESHOLD_PIXELS results.append(on_target) success_rate = sum(results) / len(results) print(f"On-target rate: {success_rate:.1%}") assert success_rate >= 0.80, "Target not met -- tune Kalman parameters"

# SECTION 7: IMPLEMENTATION TIMELINE

| Week | Phase | Tasks |
| --- | --- | --- |
| Week 1 | Phase 1: Analysis | Instrument loop, measure latency, document interfaces |
| Week 2 | Phase 2: Kalman | Implement KalmanTracker, unit test with synthetic data |
| Week 3 | Phase 2: Integration | Integrate predictor + coordinate transform, connect to PTU |
| Week 4 | Phase 3: Testing | Run validation tests, tune parameters, achieve ≥80% |
| Week 5 | Phase 3: Optimization | Threading, performance profiling, final validation |

# SECTION 8: DEPENDENCIES & SETUP

## 8.1 Python Packages

pip install filterpy numpy opencv-python ultralytics

## 8.2 Key Configuration (config.py)

# Camera FRAME_WIDTH = 1920 FRAME_HEIGHT = 1080 FPS = 30 # Zoom-calibrated FOV (measure for each zoom level) H_FOV_DEG = 0.5 # 36x zoom horizontal FOV V_FOV_DEG = 0.3 # 36x zoom vertical FOV # Kalman tuning KALMAN_R = 10.0 KALMAN_Q = 0.1 # System lag (measure in Phase 1) SYSTEM_LAG_SECONDS = 0.10 # On-target threshold ON_TARGET_PIXEL_RADIUS = 20

# SECTION 9: RISKS & MITIGATIONS

| Risk | Mitigation |
| --- | --- |
| FOV calibration error at 36x zoom | Calibrate FOV per zoom level using known target distance |
| Kalman diverges on fast maneuvers | Increase Q (process noise) to track sudden acceleration |
| YOLO detection gaps > 0.5s | Implement coast mode with velocity decay |
| PTU command latency spike | Use non-blocking PTU commands; measure and compensate |
| Python GIL limits throughput | Separate YOLO to subprocess with multiprocessing.Queue |

END OF DOCUMENT